# Phaeno Barcode & DataMatrix Plan

Recommended architecture for tube identification, scanning, printing, and duplicate-ID prevention

Purpose: establish a simple, durable barcode standard for Phaeno laboratory operations and the PSeq Operations System (POS). The core principle is that the barcode identifies the physical container; the database determines what biological material is in that container.

# 1. Recommended Standard

Use manufacturer-prebarcoded tubes whenever practical, especially for long-term sample storage.

Standardize on 2D DataMatrix for small tubes and other compact lab containers.

Use thermal-transfer printing for Phaeno-created labels; use 300 dpi or higher for small DataMatrix symbols.

Treat every barcode as an immutable container identifier, not as a carrier for patient, project, sample type, or other changing metadata.

Make the PSeq Operations System database the authoritative source for container identity.

# 2. What DataMatrix Is

DataMatrix is a compact two-dimensional barcode designed to encode information in a small square or rectangular symbol. It is well suited to laboratory tubes because it can be printed very small, supports error correction, and can be read from different orientations.

For Phaeno, the DataMatrix should normally contain only a short identifier, for example:

PH-T-00001847

The DataMatrix is simply the machine-readable representation of that string. The POS application does not need to understand the graphical pattern; the scanner decodes it and sends the identifier to the application.

# 3. Container Identity vs. Sample Identity

Keep physical container identity separate from biological sample identity. This makes aliquoting, transfers, storage moves, chain-of-custody, and future automation much easier.

| Container | Sample / Content |
| --- | --- |
| Internal ContainerId (UUID) | SampleId / AccessionId |
| Barcode(s) | Sample type |
| Container type | Aliquot identity |
| Manufacturer / lot | Quantity / concentration |
| Status | Customer / project |
| Current location | Other biological metadata |

# 4. Recommended POS Data Model

## Container

Use an internal UUID as the database primary key. Do not use the printed barcode itself as the database primary key.

Id (UUID)

ContainerType

Manufacturer

ManufacturerLot

Status

CreatedAt

## ContainerBarcode

Id

ContainerId

Namespace

Value

Symbology

Source

IsPrimary

This model allows a container to carry a manufacturer barcode, a Phaeno-applied barcode, or both without changing the core container record.

# 5. Barcode Namespace and Duplicate Prevention

There are two distinct duplicate risks: duplicates among Phaeno-generated identifiers and collisions between identifiers supplied by different manufacturers.

## Phaeno-generated IDs

The server/database should generate or reserve the identifier before printing. React or the printer should never independently invent a tube ID. A database uniqueness constraint prevents two operators or processes from receiving the same identifier even when they create containers simultaneously.

Example: UNIQUE (Namespace, Value)

## Manufacturer IDs

Do not assume that every manufacturer's barcode namespace is globally coordinated. Store the barcode namespace together with the barcode value.

| Namespace | Value |
| --- | --- |
| PHAENO | PH-T-00001847 |
| EPPENDORF | 003847192847 |
| AZENTA | 003847192847 |

The same numeric value can therefore exist in two manufacturer namespaces without being treated as the same physical identifier.

# 6. Printing Workflow

Recommended sequence for Phaeno-created labels:

Operator requests a new container in POS.

Server creates the Container record.

Database generates/reserves the Phaeno barcode value.

POS generates a DataMatrix symbol containing that value.

Label is printed using a 300 dpi or higher thermal-transfer printer and an appropriate laboratory-grade label/ribbon combination.

Operator applies the label to the tube.

Operator scans the newly printed label back into POS.

POS verifies that the scanned value matches the barcode reserved for that container.

Container becomes active.

# 7. Scanning Workflow

Most 2D USB scanners can operate in USB HID keyboard mode. In this mode, scanning a DataMatrix behaves like rapidly typing the decoded identifier followed by Enter. This allows the browser-based POS to support scanning without a vendor-specific SDK.

Typical receiving flow:

Open Receive Samples.

Scan the tube.

POS receives the decoded barcode string.

POS searches ContainerBarcode using the appropriate namespace/value.

If found, display the container and associated sample information.

If unknown, offer the controlled workflow to register the container.

# 8. Lab Movement Pattern

Design scanning as a first-class interaction throughout POS. A useful general pattern is:

SCAN OBJECT  ->  SCAN DESTINATION  ->  CONFIRM

Examples include receiving a tube, moving it to a freezer location, transferring it to a rack, creating an aliquot, or recording a processing step. This pattern provides a strong basis for inventory control and chain-of-custody.

# 9. Equipment Guidance

2D scanner: must explicitly support DataMatrix; avoid 1D-only barcode scanners.

Printer: thermal-transfer rather than ordinary direct-thermal printing.

Resolution: 300 dpi minimum for small tube labels.

Labels/ribbons: choose laboratory-grade materials appropriate for expected temperature, moisture, solvents, and storage duration.

Prebarcoded tubes: preferred where practical because the permanent identifier is applied during manufacturing and reduces local labeling steps.

# 10. Implementation Priorities

| Priority | Capability | Recommendation |
| --- | --- | --- |
| 1 | Data model | Container + ContainerBarcode + sample/content relationship |
| 2 | Uniqueness | Database-enforced Namespace + Value uniqueness |
| 3 | Scanning | 2D DataMatrix scanner in HID keyboard mode |
| 4 | Receiving | Scan and resolve existing/unknown container |
| 5 | Printing | Server-reserved ID -> DataMatrix -> thermal-transfer label |
| 6 | Verification | Mandatory scan-back for newly printed labels |
| 7 | Movement | Scan object -> scan destination -> confirm |

# 11. Recommended Phaeno Rule

The barcode identifies the container. The database identifies what is in the container.

Keeping that rule consistent across POS will prevent the barcode scheme from becoming coupled to changing sample metadata and will leave room for future automation, robotics, racks, plates, and a broader laboratory operations system.